ПОЛНАЯ ИСПРАВЛЕННАЯ КОНФИГУРАЦИЯ ПО МЕТОДИЧКЕ СиСА 2026

ISP

hostnamectl

set-hostname

isp

exec

bash

# WAN (магистраль)

mcedit

/etc/net/ifaces/ens3/options

TYPE

=

eth

BOOTPROTO

=

dhcp

# HQ

mkdir

-

p

/etc/net/ifaces/ens4

echo

"172.16.1.1/28"

>

/etc/net/ifaces/ens4/ipv4address

mcedit

/etc/net/ifaces/ens4/options

TYPE

=

eth

BOOTPROTO

=

static

# BR

mkdir

-

p

/etc/net/ifaces/ens5

echo

"172.16.2.1/28"

>

/etc/net/ifaces/ens5/ipv4address

mcedit

/etc/net/ifaces/ens5/options

TYPE

=

eth

BOOTPROTO

=

static

# IP Forward

mcedit

/etc/net/sysctl.conf

net.ipv4.ip_forward

=

1

systemctl

restart

network

apt-get

update

&&

apt-get

install

iptables

-

y

iptables

-

t

nat

-

A

POSTROUTING

-

s

172

.16.1.0/28

-

o

ens3

-

j

MASQUERADE

iptables

-

t

nat

-

A

POSTROUTING

-

s

172

.16.2.0/28

-

o

ens3

-

j

MASQUERADE

iptables-save

>

/etc/sysconfig/iptables

systemctl

enable

--

now

iptables

timedatectl

set-timezone

Europe/Moscow

HQ-RTR

enable

configure

terminal

hostname

HQ-RTR

ip

domain-name

au-team.irpo

username

net_admin

password

P@ssw0rd

role

admin

interface

isp

ip

address

172

.16.1.2/28

ip

nat

outside

exit

interface

vl100

ip

address

10

.10.100.1/27

ip

nat

inside

exit

interface

vl200

ip

address

10

.10.200.1/27

ip

nat

inside

exit

interface

vl999

ip

address

10

.10.30.1/29

ip

nat

inside

exit

port

ge1

service-instance

ge1/vl100

encapsulation

dot1q

100

exact

rewrite

pop

1

connect

ip

interface

vl100

exit

service-instance

ge1/vl200

encapsulation

dot1q

200

exact

rewrite

pop

1

connect

ip

interface

vl200

exit

service-instance

ge1/vl999

encapsulation

dot1q

999

exact

rewrite

pop

1

connect

ip

interface

vl999

exit

interface

tunnel.0

description GRE-to-BR-RTR ip address 10.10.10.1/30 ip tunnel 172.16.1.2 172.16.2.2 mode gre ip ospf authentication message-digest ip ospf message-digest-key 1 md5 P@ssw0rd exit

router ospf 1

ospf router-id 10.10.10.1 passive-interface default no passive-interface tunnel.0 network 10.10.100.0/27 area 0 network 10.10.200.0/27 area 0 network 10.10.30.0/29 area 0 network 10.10.10.0/30 area 0 exit

ip nat pool HQ 10.10.30.0-10.10.200.255

ip nat source dynamic inside-to-outside pool HQ overload interface ISP

no dhcp-server 1

ip pool VLAN200 10.10.200.2-10.10.200.30

dhcp-server 1

pool VLAN200 1

mask 255.255.255.224 gateway 10.10.200.1 dns 10.10.100.2 domain-name au-team.irpo

interface vl200

dhcp-server 1 exit

ntp timezone utc+3 write memory

HQ-SRV (DNS/BIND9)

hostnamectl

set-hostname

hq-srv.au-team.irpo

mkdir

-

p

/etc/net/ifaces/ens3

echo

"10.10.100.2/27"

>

/etc/net/ifaces/ens3/ipv4address

echo

"default via 10.10.100.1"

>

/etc/net/ifaces/ens3/ipv4route

mcedit

/etc/net/ifaces/ens3/options

TYPE

=

eth

BOOTPROTO

=

static

systemctl restart network

useradd sshuser -u 2026 echo 'sshuser:P@ssw0rd' | chpasswd gpasswd -a sshuser wheel

mcedit /etc/sudoers %wheel ALL=(ALL:ALL) NOPASSWD: ALL

mcedit /etc/openssh/sshd_config

Port 2026

MaxAuthTries 2

Banner /etc/openssh/banner AllowUsers sshuser

echo "Authorized access only" > /etc/openssh/banner systemctl restart sshd apt-get update && apt-get install bind bind-utils -y

mcedit /var/lib/bind/etc/options.conf listen-on port 53 { 10.10.100.2; any; }; forwarders { 77.88.8.8; }; allow-query { any; };

mcedit /var/lib/bind/etc/rfc1912.conf zone "au-team.irpo" { type master; file "au-team.irpo";

};

zone "100.10.10.in-addr.arpa" { type master;

file "10.10.100.in-addr.arpa";

};

zone "200.10.10.in-addr.arpa" { type master;

file "10.10.200.in-addr.arpa";

};

cd /var/lib/bind/etc/zone cp empty au-team.irpo cp empty 10.10.100.in-addr.arpa cp empty 10.10.200.in-addr.arpa

# ПРЯМАЯ ЗОНА mcedit au-team.irpo

$TTL 1D

@ IN SOA hq-srv.au-team.irpo. root.au-team.irpo. (

2026051001 12H 1H 1W 1H )

@	IN NS	hq-srv.au-team.irpo.

hq-rtr IN A	10.10.100.1 br-rtr IN A	10.20.20.1 hq-srv IN A	10.10.100.2 hq-cli IN A	10.10.200.2

br-srv IN A	10.20.20.2 docker IN A	172.16.1.1 web	IN A	172.16.2.1

# ОБРАТНАЯ VLAN100 mcedit 10.10.100.in-addr.arpa 1 IN PTR hq-rtr.au-team.irpo.

2 IN PTR hq-srv.au-team.irpo.

# ОБРАТНАЯ VLAN200 mcedit 10.10.200.in-addr.arpa 1 IN PTR hq-rtr.au-team.irpo. 2 IN PTR hq-cli.au-team.irpo.

rndc-confgen > /etc/bind/rndc.key sed -i '6,$d' /etc/bind/rndc.key chown -R root:named /var/lib/bind/etc/zone/*

named-checkconf named-checkconf -z systemctl enable --now bind

echo "search au-team.irpo" > /etc/resolv.conf echo "nameserver 10.10.100.2" >> /etc/resolv.conf timedatectl set-timezone Europe/Moscow

HQ-CLI

hostnamectl

set-hostname

hq-cli.au-team.irpo

mkdir

-

p

/etc/net/ifaces/ens3

mcedit

/etc/net/ifaces/ens3/options

TYPE

=

eth

BOOTPROTO

=

dhcp

echo

"search au-team.irpo"

>

/etc/resolv.conf

echo

"nameserver 10.10.100.2"

>>

/etc/resolv.conf

systemctl

restart

network

timedatectl

set-timezone

Europe/Moscow

BR-RTR

enable

configure

terminal

hostname

BR-RTR

ip

domain-name

au-team.irpo

username

net_admin

password

P@ssw0rd

role

admin

interface

isp

ip

address

172

.16.2.2/28

ip

nat

outside

exit

interface

br

ip

address

10

.20.20.1/28

ip

nat

inside

exit

port

te1

service-instance

te1/br

encapsulation

untagged

connect

ip

interface

br

exit

interface

tunnel.0

description

GRE-to-HQ-RTR

ip

address

10

.10.10.2/30

ip

tunnel

172

.16.2.2

172

.16.1.2

mode

gre

ip

ospf

authentication

message-digest

ip

ospf

message-digest-key

1

md5

P@ssw0rd

exit

router

ospf

1

ospf

router-id

10

.10.10.2

passive-interface

default

no

passive-interface

tunnel.0

network

10

.20.20.0/28

area

0

network

10

.10.10.0/30

area

0

exit

ip

nat

source

dynamic

inside-to-outside

interface

isp

overload

ip

route

0

.0.0.0/0

172

.16.2.1

ntp

timezone

utc+3

write

memory

ip nat pool BR-POOL 172.16.2.2 172.16.2.2ssss

ip nat source dynamic inside-to-outside pool BR-POOL overload interface isp

BR-SRV

hostnamectl

set-hostname

br-srv.au-team.irpo

mkdir

-

p

/etc/net/ifaces/ens3

echo

"10.20.20.2/28"

>

/etc/net/ifaces/ens3/ipv4address

echo

"default via 10.20.20.1"

>

/etc/net/ifaces/ens3/ipv4route

mcedit

/etc/net/ifaces/ens3/options

TYPE

=

eth

BOOTPROTO

=

static

useradd

sshuser

-

u

2026

echo

'sshuser:P@ssw0rd'

|

chpasswd

gpasswd

-

a

sshuser

wheel

echo

"search au-team.irpo"

>

/etc/resolv.conf

echo

"nameserver 10.10.100.2"

>>

/etc/resolv.conf

systemctl

restart

network

timedatectl

set-timezone

Europe/Moscow